Ministry of Higher Education							       Time Allowed: 2 Hours

Cairo Higher Institute									Open-Book/Notes

Computer and Electronic Systems Engineering Department

Microprocessors and Interfacing Final Exam

January 2012

# Solve As Much As You Can – Maximum Grade: 25 Points

# Part 1. Answer the following questions by marking the best answer among the choices given:  [3 points each]

- For high time accuracy applications, a clock source for C8051F020 based on … is preferred.

  - RC oscillator

  - Crystal oscillator (*)

  - Internal clock

- Accessing C8051F020 memory location 083H using direct addressing mode refers to …

  - Special function register  (*)

  - General purpose data RAM memory address

  - Immediate value

- GPIO pins can be used for bidirectional data transfer when they are configured as …

  - Open-drain mode with internal weak pull-ups (*)

  - Open-drain mode

  - Push-pull mode

- In asynchronous communication, the transmitter and receiver share …

- Common clock

- Common serial data  (*)

- Common clock and serial data

- When timer 2 overflow interrupt is serviced, the microcontroller executes interrupt service routine from address …

- 0x002B (*)

- 0x0023

- 0x000B

- In synchronous communication, the transmitter and receiver share …

- Common clock

- Common serial data  (*)

- Common clock and serial data

- The throughput of serial data sent at 9600 baud, using 8-bit data, 1 start, 1 stop, and 1 parity bit is …

  - 6981 bits/s (*)

  - 7674 bits/s

  - 9103 bits/s

- When timer 3 overflow interrupt is serviced, the microcontroller executes interrupt service routine from address …

- 0x00A3

- 0x009B

- 0x0073 (*)

- Indexed addressing mode is useful in …

  - Jump or branching instructions

  - Data transfer from a look-up table (*)

  - Arithmetic instructions.

- For human interface devices based on C8051F020, a system clock based on … is preferred.

  - RC oscillator

  - Crystal oscillator

  - Internal oscillator (*)

# Part 2. Mark the following statement as either True (T) or False (F): [1.5 points each]

- One can declare bit-addressable variables in the data memory of C8051F020. (T)

- To detect errors in serial communication, a parity bit should be added. (T)

- Different c8051F020 designs may configure the external pins to have different use. (T)

- Timers 2 and 4 have identical structure but timer 4 has a higher interrupt priority. (F)

- The RESET interrupt (interrupt 0) cannot be turned off. (T)

- Auto-reload timers do not need to be configured and started every time an overflow occurs. (T)

- Timer 1 mode 1 can be used to obtain longer timing periods than Timer 1 mode 0. (T)

- Timer overflow flag is automatically cleared by the microcontroller with no program intervention. (F)

- Timer 1 mode 3 allows for two 8-bit timers to be available. (F)

- It is possible to configure a 16-bit timer with auto-reload in c8051F020. (T)

- Relative addressing is rarely used in C8051F020 assembly language programming. (F)

- All GPIO ports are bit addressable. (F)

- To correct errors in serial communication, the parity bit can be used to do that. (F)

- The Timer 0 interrupt cannot be turned off. (F)

- Timer 3 has only one mode of operation. (T)

# Part 3. Denote the following C8051F020 microcontroller instructions as either being True (T) or False (F) assembly instructions: [1.5 points each]

- ACALL @R1 		(F)

- CJNE	R3,#50H,0F0H (T)

- MOV 010H, #010H (T)

- MOV @R1, 030H (T)

- ADD A, 030H (T)

- RLC #040H (F)

- DJNZ R7, 70H (T)

- CPL P2.4 (T)

- MAIN EQU 070H (F)

- POP 070H (T)

- XCH A, #05AH (F)

- XCH A, @R0 (T)

- MOV C, P1.1 (T)

- ORL C, /OV  (T)

- CJNE R6, 70H (F)

- CPL P1^3 (F)

- SETB P2.0 (T)

- INC @R1 (T)

- MUL A, B (F)

- CJNE R6, 70H

- MOV DPTR, #01160H (T)

- MOV @R0, A		(T)

- ADD A, @030H (F)

- DJNZ R7, 70H (T)

- CPL P5.4	(F)

- POP 070H (T)

- CPL P1.2	(T)

- SETB C  (T)

- MOV DPTR, 01160H (F)

- SUBB	A,R0 (T)

# Part 4. Compute the output of the following operations on byte operands in a C Language program for a C8051F020 device: [1.5 points each]

- (2 | 0x40 <<2) 			0x2

- (0xF0 + 0x08) >= 0 		true

- 0x2F % 0x02 			1

- ~1 && 2 				false

- 0A5H & 05AH			0

- 0xF0<<4				0

- 0x87 % 0x08   	  		7

# Part 5. Write C8051F020 assembly code lines to do the following: [6 Points each]

c. Configuration of external crystal oscillator working at 10 MHz.

d. Generation of a periodic 100 kHz binary signal with duty cycle of 30% from pin P1.6.

e. Transfer the contents of memory location 090H to accumulator A.

Part 6. [10 points] Write a simple program that would make a LED at P5.5 blink with an on time of 0.5s and off time of 0.3s when an input button at P5.3 is pressed and stay off otherwise.

Part: Based on the Priority Crossbar Decode Table, state what would happen in the following cases by showing on the table below the locations of the peripherals that will appear on the c8051F020 port pins:

- Writing XBR0=0x06, XBR1=0, then XBR2=0x44 (in sequence).

- Writing XBR0=0x06, XBR1=0, then XBR2=0x04 (in sequence).

- Writing XBR2=0x44, XBR1=0, then XBR2=0x06 (in sequence).

Part:

- If we want SDA to be available on pin P0.2, write the cross-bar initialization to do that.

- Write a c8051F020 code to enable timer 3 interupt.

- Write a c8051F020 code to enable timer 4 interupt.

- Write a c8051F020 code to initialize Timer 0 mode 1 with an internal clock/12 and the maximum period.

- Write a c8051F020 code to initialize Timer 4 as a 16-bit timer with auto-reload with an internal clock undivided and the maximum period.

- Write a c8051F020 code to initialize the clock source to be an external crystal oscillator with frequency 100kHz.

- Write a c8051F020 code to initialize the clock source to be an internal crystal oscillator with frequency 4 MHz.