Ministry of Higher Education							       Time Allowed: 2 Hours

Cairo Higher Institute									Open-Book/Notes

Computer and Electronic Systems Engineering Department

Advanced Topics: Embedded Systems Final Exam

January 2012

# Solve As Much As You Can – Maximum Grade: 25 Points

# Part 1. Answer the following questions by marking the best answer among the choices given:  [3 points each]

- Accessing C8051F020 memory location 083H using indirect addressing mode refers to …

  - Special function register

  - General purpose data RAM memory address (*)

  - Immediate value

- We can toggle bit 0 of P5 by the following C instruction …

  - P5^= 001H (*)

  - P5&= 0FEH

  - P5.0= ~P5.0

- GPIO pins can be used for bidirectional data transfer when they are configured as …

  - Open-drain mode with internal weak pull-ups (*)

  - Open-drain mode

  - Push-pull mode

- When the external crystal oscillator is invalid for a long period of time, the microcontroller  …

  - Switches automatically to internal oscillator

  - Can be configured to reset (*)

  - Causes a flag to be raised for the program to repair the problem

- The instruction to be used to transfer data from external data memory is …

  - MOV

  - MOVX (*)

  - MOVC

- In asynchronous communication, the transmitter and receiver share …

- Common clock

- Common serial data  (*)

- Common clock and serial data

- The throughput of serial data sent at 28,800 baud, using 8-bit data, 1 start, 1 stop, and no parity is …

- 22100 bits/s

- 23040 bits/s (*)

- 25480 bits/s

11. When timer 2 overflow interrupt is serviced, the microcontroller executes interrupt service routine from address …

- 0x002B (*)

- 0x0023

- 0x000B

- In synchronous communication, the transmitter and receiver share …

- Common clock

- Common serial data

- Common clock and serial data (*)

- The throughput of serial data sent at 9600 baud, using 8-bit data, 1 start, 1 stop, and 1 parity bit is …

  - 6981 bits/s (*)

  - 7674 bits/s

  - 9103 bits/s

- When using a reference voltage of 1.2V, the output of DAC0 for an input digital value of 512 is …

- 0.5 V

- 0.3 V

- 0.15 V (*)

- When using a reference voltage of 2.4V, the output of ADC0 for an input voltage of 0.3V is …

- 0.5 V

- 0.3 V

- 0.15 V (*)

# Part 2. Mark the following statement as either True (T) or False (F): [1.5 points each]

- C8051F020 is a mixed-signal microcontroller because it can handle analog and digital inputs.  (T)

- The C8051F020 instruction MOV cannot be used with indirect addressing.  (F)

- Microcontroller external clock configuration must perform a check on the external clock validity. (T)

- Division must work only in direct addressing mode in c8051F020 assembly language. (F)

- Using sbit to declare a bit variable is valid only for local variables. (F)

- 8-bit MCUs are not suitable for low-power applications. (F)

- It is not possible to declare a bit array variable in C language programming for c8051F020. (T)

- The operands of a logical operation must be Boolean. (T)

- Data in C8051F020 are 16-bit wide. (F)

- Not all GPIO ports are bit addressable. (T)

- To correct errors in serial communication, the parity bit can be used to do that. (F)

- Peripherals inside the microcontroller can be made available on external pins using watchdog. (F)

- Timers 2 and 4 have identical structure but timer 2 has a higher interrupt priority. (T)

- The RESET interrupt (interrupt 0) cannot be turned off. (T)

- Timer 0 mode 1 can be used to obtain longer timing periods than Timer 0 mode 0. (T)

- Auto-reload timers need to be configured and started every time an overflow occurs. (F)

- Timer overflow flag must be cleared by program to ensure proper operation. (T)

- Timers 0 and 1 working in mode 3 allow for four 8-bit timers to be available. (F)

- Timer 3 has only one mode of operation. (T)

- It is possible to configure a 13-bit timer with capture in c8051F020. (F)

# Part 3. Denote the following C8051F020 microcontroller instructions as either being True (T) or False (F) assembly instructions: [1.5 points each]

- CPL P1^3 (F)

- SETB P2.0 (T)

- INC @R1 (T)

- MUL A, B (F)

- CJNE R6, 70H (F)

- MOV DPTR, #01160H (T)

- MOV @R0, A		(T)

- ADD A, @030H (F)

- DJNZ R7, 70H (T)

- CPL P5.4	(F)

- POP 070H (T)

- CPL P1.2	(T)

- SETB C  (T)

- MOV DPTR, 01160H (F)

- SUBB	A,R0 (T)

# Part 4. Compute the output of the following operations on byte operands in a C Language program for a C8051F020 device: [1.5 points each]

- (1 ^ 0x40 <<1) 			 0x81

- ~1 && 2 				 False

- 0A5H & 05AH			0

- 0xF0<<4				0

- 0x87 % 0x08   	  		7

# Part 5. Write a C8051F020 code to do the following: [6 Points each]

- Make SDA available on pin P0.6.

- Write a c8051F020 code to enable timer 3 interrupt.

- Initialize Timer 4 as a 16-bit timer with auto-reload with an internal clock undivided and the maximum period possible.

- Initialize the clock source to be an external crystal oscillator with frequency 100kHz.

Part 6. [10 points] Based on the Priority Crossbar Decode Table, state what would happen in the following cases by showing on the table below the locations of the peripherals that will appear on the c8051F020 port pins:

- Writing XBR0=0x06, XBR1=0, then XBR2=0x44 (in sequence).

- Writing XBR0=0x06, XBR1=0, then XBR2=0x04 (in sequence).

- Writing XBR2=0x44, XBR1=0, then XBR2=0x06 (in sequence).

Best of Luck!